# Converted from PDF

APPENDIX A – SCOTT SCHEUDLE 
 
Claimant: M Trevor Matthews 
Property Address: 10 Cherwell House, Milton Keynes, MK3 7BJ  
Item 
Detail of Disrepair 
Breach – all 
repair 
obligations fall 
under section 
9a and 11 of the landlord and tenant act 1985

Assessment of Remedial 
Works Required 
Estimate of 
costs (These do 
not include 
Contractor’s 
Preliminaries)

Defendant’s Assessment 
Comments 
Defendant’s Assessment of 
Remedial Work Required 
Defendant’s Assessment of Costs

1. - Lounge and 
bedroom have 
damp, mould and 
high humidity. 
 
- No insulation in 
the walls which is 
causing property 
to freeze. 
 
- Windows are 
poor quality letting 
cold in. 
 
- Mould all around 
windows and 
damp on property. 
 
- Gas pipes 
coming into living 
room which is a 
tripping hazard. 
 
- Broken pipe 
near boiler. 
 
- Loud noise from 
neighbours. 
The writer can 
confirm that there 
is no significant 
dampness in the 
lounge or 
bedroom.  
No valid claim. 
No remedial work required. 
£0.00 
  
  
  
The writer can 
confirm that there 
is no significant 
mould in the living 
room.  
No valid claim. 
No remedial work required. 
£0.00 
  
  
  The writer can 
confirm that there 
is significant 
mould in the 
bedroom 
especially around 
the window. 
Both Sections 
9a and 11 
Wash down mould patches 
with fungicide and apply 2no. 
coats mould resistant paint to 
wall. 
£60.96 
  
  
  The writer can 
confirm that there 
is insufficient 
ventilation in the 
property.    
Both Sections 
9a and 11 
Install an extractor fan with 
humidistat control in the 
bathroom. 
£225.00 
  
  
  
Install an extractor fan in the 
kitchen. 
£225.00 
  
  
  
The writer can 
confirm that the 
walls are not 
insulated to 
current standards 
but that 
No valid claim. 
No remedial work required. 
£0.00 
  
  
improvements in 
this regard will not 
be technically 
feasible or 
economical.     
The writer can 
confirm that the 
windows are 
somewhat dated 
but are double 
glazed and 
provided with 
trickle vents.  
They are in 
satisfactory 
condition. 
No valid claim. 
No remedial work required. 
£0.00 
  
  
  
The writer can 
confirm that there 
is no gas pipe trip 
hazard in the 
living room.  This 
was repaired by 
the landlord some 
time ago.  
No valid claim. 
No remedial work required. 
£0.00 
  
  
  
The writer can 
confirm that there 
is no broken pipe 
by the boiler.  This 
was also repaired 
by the landlord 
some time ago.  
No valid claim. 
No remedial work required. 
0.00 
  
  
TOTAL 
£510.96 
PLUS VAT